Router configuration — Router-on-a-Stick (subinterfaces)

We will make subinterfaces on R1 (Router 2911) G0/0 interface — one subinterface for each VLAN. The physical link from R1 G0/0 must be connected to SW0 on the port set to trunk (we used SW0 Fa0/3 as trunk earlier).

Goals:

- Configure .1 IP per VLAN as gateway.

- Use encapsulation dot1Q to label frames with VLAN IDs.

- Bring interfaces up.

R1 configuration (full commands + explanation)

- R1> enable

- R1# configure terminal

! Ensure the physical interface is up first

- R1(config)# interface gigabitEthernet 0/0

- R1(config-if)# no shutdown

- R1(config-if)# exit

! Subinterface for HR VLAN 10

- R1(config)# interface gigabitEthernet 0/0.10

- R1(config-subif)# description HR VLAN 10 subinterface

- R1(config-subif)# encapsulation dot1Q 10

- R1(config-subif)# ip address 192.168.10.1 255.255.255.0

- R1(config-subif)# exit

! Subinterface for IT VLAN 20

- R1(config)# interface gigabitEthernet 0/0.20

- R1(config-subif)# description IT VLAN 20 subinterface

- R1(config-subif)# encapsulation dot1Q 20

- R1(config-subif)# ip address 192.168.20.1 255.255.255.0

- R1(config-subif)# exit

! Subinterface for Sales VLAN 30

- R1(config)# interface gigabitEthernet 0/0.30

- R1(config-subif)# description SALES VLAN 30 subinterface

- R1(config-subif)# encapsulation dot1Q 30

- R1(config-subif)# ip address 192.168.30.1 255.255.255.0

- R1(config-subif)# exit

! Save config

- R1(config)# end

- R1# copy running-config startup-config

Explain each important line:

- interface gigabitEthernet 0/0.10 creates logical subinterface .10 on the physical G0/0.

- encapsulation dot1Q 10 tells the router to accept/tag frames with VLAN ID 10 on this subinterface.

- ip address 192.168.10.1 255.255.255.0 places the default gateway for VLAN 10 on this subinterface.

- Do not put an IP on the physical g0/0 when using subinterfaces (unless you want a native VLAN address). Use subinterfaces for each VLAN.