Router-on-a-Stick Configuration Commands

- enable

- configure terminal

! --- Make sure physical interface is up ---

interface gigabitEthernet 0/0

no shutdown

description Connection-to-SW0 (Trunk)

exit

! --- HR VLAN 10 subinterface ---

interface gigabitEthernet 0/0.10

description HR VLAN 10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

! --- IT VLAN 20 subinterface ---

interface gigabitEthernet 0/0.20

description IT VLAN 20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

! --- Sales VLAN 30 subinterface ---

interface gigabitEthernet 0/0.30

description Sales VLAN 30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

exit

! --- Save the configuration ---

end

copy running-config startup-config

🔹 Explanation of Each Command

| Command | Purpose |
| --- | --- |
| interface gigabitEthernet 0/0 | Selects the physical interface connected to SW0. |
| no shutdown | Activates the interface (by default, new interfaces are shutdown). |
| interface gigabitEthernet 0/0.10 | Creates a subinterface for VLAN 10 (HR). |
| encapsulation dot1Q 10 | Tells router this subinterface handles VLAN 10 tagged traffic. |
| ip address 192.168.10.1 255.255.255.0 | Assigns default gateway for VLAN 10. |
| Repeat for .20 and .30 | Same logic for IT (VLAN 20) and Sales (VLAN 30). |
| copy running-config startup-config | Saves config so it persists after reboot. |

⚠️ Important: Do not assign an IP on the physical G0/0 interface. Only subinterfaces get IPs when using Router-on-a-Stick.

🔹 Verification Steps

1️⃣ Check interfaces and IPs

R1# show ip interface brief

✅ Should show:

G0/0            unassigned      administratively up   down

G0/0.10         192.168.10.1   up                    up

G0/0.20         192.168.20.1   up                    up

G0/0.30         192.168.30.1   up                    up

2️⃣ Check routing table (connected networks)

R1# show ip route

✅ Should show:

C 192.168.10.0/24 is directly connected, GigabitEthernet0/0.10

C 192.168.20.0/24 is directly connected, GigabitEthernet0/0.20

C 192.168.30.0/24 is directly connected, GigabitEthernet0/0.30