Switch Trunk Port Configuration (No VLAN yet)

Let’s prepare trunk links first, to avoid future inter-VLAN routing issues.

On all three switches (do this now before VLAN creation):

On SW0 (core):

- enable

- configure terminal

- interface fa0/1

- switchport mode trunk

- switchport trunk encapsulation dot1q

- exit

- interface range fa0/23-24

- switchport mode trunk

- switchport trunk encapsulation dot1q

- exit

On SW1:

- enable

- configure terminal

- interface fa0/24

- switchport mode trunk

- switchport trunk encapsulation dot1q

- exit

On SW2:

- enable

- configure terminal

- interface fa0/23

- switchport mode trunk

- switchport trunk encapsulation dot1q

- exit

✅ Why this matters:
If you skip trunk setup before VLAN configuration, VLAN tags might not pass between switches, causing “inter-VLAN routing not working” later.