Perfect! Let’s go step by step and configure ACLs on R1 in Packet Tracer, with a detailed explanation so there are no mistakes.

Step 1: ACL Configuration – Detailed Guide

Objective

- Restrict HR VLAN (VLAN 10) from accessing IT VLAN (VLAN 20)

- Allow all other traffic

- ACL will be applied on Router R1 (Router-on-a-Stick)

We’ll use a standard/extended IP access-list to filter traffic.

Step 1.1: Access Router CLI

- Click on Router R1 in Packet Tracer

- Go to the CLI tab

- Enter privileged mode:

enable

- Prompt changes from > to #

Step 1.2: Enter Global Configuration Mode

configure terminal

- Prompt changes to (config)#

- Now you can configure ACLs, interfaces, etc.

Step 1.3: Create Extended ACL

We want to deny HR → IT and allow everything else.

! Create ACL 100

access-list 100 deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

access-list 100 permit ip any any

Explanation

| Command | Meaning |
| --- | --- |
| access-list 100 | Creates extended ACL #100 |
| deny ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255 | Deny any traffic from HR subnet to IT subnet |
| permit ip any any | Allow all other traffic (important!) |

⚠️ Important: ACLs have an implicit “deny all” at the end, so you must explicitly permit other traffic, otherwise everything else will be blocked.

Step 1.4: Apply ACL to the Router Interface

- We are using Router-on-a-Stick, so each VLAN has a subinterface

- Apply ACL outgoing on HR VLAN subinterface:

interface g0/0.10

ip access-group 100 out

exit

Explanation

| Command | Meaning |
| --- | --- |
| interface g0/0.10 | Enter subinterface for VLAN 10 (HR) |
| ip access-group 100 out | Apply ACL #100 to traffic leaving HR VLAN |
| exit | Go back to global config |

Why outgoing:

- Traffic leaving HR towards IT is filtered.

- Other VLAN traffic (e.g., HR → Sales) is unaffected because ACL allows it.

Step 1.5: Save Configuration

end

copy running-config startup-config

- Ensures ACL stays after router reboot

Step 1.6: Test ACL

- Go to HR PC → Desktop → Command Prompt

- Test ping IT PC (should fail):

ping 192.168.20.10

- Test ping Sales PC (should succeed):

ping 192.168.30.10

✅ Expected Result:

- HR → IT → fail

- HR → Sales → success

- Other inter-VLAN traffic works

Step 1.7: Verification Commands on Router

show access-lists

show ip interface g0/0.10

- Check ACL rules applied and hit counts

- Hit counts increase when traffic matches ACL

✅ Summary

- ACL 100 denies HR → IT and allows all other traffic

- Applied outgoing on HR VLAN subinterface

- Tested pings confirm rules

- Configuration saved