1️⃣ DHCP Configuration on Router (R1)

We’ll configure automatic IP assignment for each VLAN. This eliminates the need to manually set static IPs for PCs.

R1 CLI Commands (Copy & Paste)

enable

configure terminal

! --- HR VLAN 10 DHCP ---

ip dhcp pool HR

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 192.168.20.100

exit

! --- IT VLAN 20 DHCP ---

ip dhcp pool IT

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 192.168.20.100

exit

! --- Sales VLAN 30 DHCP ---

ip dhcp pool SALES

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 192.168.20.100

exit

! --- Save configuration ---

end

copy running-config startup-config

✅ Explanation:

- ip dhcp pool <name> → creates DHCP pool for VLAN

- network → subnet for DHCP clients

- default-router → default gateway for VLAN

- dns-server → DNS IP for PCs

- ip dhcp excluded-address → prevents router IPs from being assigned to PCs

2️⃣ Assign PCs to Use DHCP

For each PC in Packet Tracer:

- Desktop → IP Configuration

- Select “DHCP” instead of static IP

- PC will automatically receive an IP from its VLAN pool

Example Assigned IPs (automatically):

| VLAN | PC | DHCP IP Example | Gateway |
| --- | --- | --- | --- |
| HR | PC3 | 192.168.10.10 | 192.168.10.1 |
| HR | PC4 | 192.168.10.11 | 192.168.10.1 |
| IT | PC5 | 192.168.20.10 | 192.168.20.1 |
| IT | PC6 | 192.168.20.11 | 192.168.20.1 |
| Sales | PC1 | 192.168.30.10 | 192.168.30.1 |
| Sales | PC2 | 192.168.30.11 | 192.168.30.1 |

3️⃣ DNS Server Setup in Packet Tracer

- Add a Server to the network (e.g., connect to SW0).

- On Server → Desktop → IP Configuration:

- IP Address: 192.168.20.100

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.20.1

- On Server → Services → DNS → Add Entry:

- Name: webserver.server.local

- IP: 192.168.20.100

- PCs configuration: Desktop → IP Configuration → set DNS server → 192.168.20.100

4️⃣ Inter-VLAN Ping Tests (After DHCP + DNS)

On any PC:

! Ping own gateway

ping <your VLAN gateway>

! Ping PC in another VLAN

ping <other VLAN PC IP>

! Ping using DNS hostname

ping webserver.server.local

✅ Expected Results:

- HR → IT, Sales → HR → IT → Sales → all should succeed

- ping webserver.server.local → resolves to 192.168.20.100 and replies

5️⃣ Verification Commands on Router

! Show assigned DHCP leases

show ip dhcp binding

! Check interface status

show ip interface brief

! Check routes

show ip route