Router: NAT Configuration — Ready-to-paste commands

Paste these into R1 CLI (exact order):

enable

configure terminal

! --- Configure public (outside) interface ---

interface gigabitEthernet 0/1

description Connection-to-Internet-Server

ip address 203.0.113.2 255.255.255.0

ip nat outside

no shutdown

exit

! --- Ensure internal interface is marked NAT inside ---

! If you previously set ip nat inside on the physical g0/0, keep it.

! For router-on-a-stick you can set it on physical interface:

interface gigabitEthernet 0/0

ip nat inside

no shutdown

exit

! --- NAT rule: permit internal networks (PAT using G0/1 IP) ---

! This ACL identifies which addresses are translated

no access-list 1

access-list 1 permit 192.168.0.0 0.0.255.255

! Overload (PAT) mapping internal addresses -> interface g0/1 IP

ip nat inside source list 1 interface gigabitEthernet 0/1 overload

! --- Save config ---

end

copy running-config startup-config

Short explanation

- ip nat inside marks internal-facing interface. Because you used subinterfaces for VLANs, marking the physical g0/0 as ip nat inside applies NAT to traffic routed from subinterfaces (this is standard practice).

- ip nat outside marks the interface connected to the Internet.

- access-list 1 permit 192.168.0.0 0.0.255.255 permits all internal private subnets (192.168.10/20/30).

- ip nat inside source list 1 interface g0/1 overload performs PAT (many private IPs -> single public IP of G0/1).

Verification commands (run after config)

Run these on R1:

show ip nat translations

show ip nat statistics

show ip interface brief

show running-config | section ip nat

show access-lists 1

What to expect:

- show ip nat translations will show active PAT entries once a PC talks to the Internet server.

- show ip nat statistics shows number of translations and hits.

- show running-config | section ip nat shows the NAT config (overload line).

- show access-lists 1 shows the ACL used by NAT.

Tests to run (on PCs)

- From any internal PC, ping the Internet server IP:

- ping 203.0.113.1

Expected: replies.

- From router, ping the Internet server:

- R1# ping 203.0.113.1

Expected: replies.

- Check NAT translations after ping from PC:

- R1# show ip nat translations

You should see ICMP or other protocol entries like tcp 192.168.10.10:xxxx -> 203.0.113.2:xxxx (depends on traffic). Packet Tracer may show icmp entries for pings.

s