🔹 SW0 (Core Switch)

enable

configure terminal

! --- VLAN Creation ---

- vlan 10

- name HR

- vlan 20

- name IT

- vlan 30

- name SALES

- exit

! --- Router connection (Fa0/1) ---

- interface fa0/1

- description Trunk-to-Router

- switchport mode trunk

- switchport trunk encapsulation dot1q

- switchport trunk allowed vlan all

- exit

! --- SW1 connection (Fa0/2) ---

- interface fa0/2

- description Trunk-to-SW1

- switchport mode trunk

- switchport trunk encapsulation dot1q

- switchport trunk allowed vlan all

- exit

! --- SW2 connection (Fa0/3) ---

interface fa0/3

- description Trunk-to-SW2

- switchport mode trunk

- switchport trunk encapsulation dot1q

- switchport trunk allowed vlan all

- exit

! --- PCs (Sales Department) ---

- interface range fa0/4 - 5

- description Sales-PCs

- switchport mode access

- switchport access vlan 30

exit

! --- Save configuration ---

- end

- write memory

🔹 SW1 (HR Department Switch)

- enable

- configure terminal

! --- VLAN Creation (optional but good practice) ---

- vlan 10

- name HR

- exit

! --- Connection to SW0 (Fa0/1) ---

- interface fa0/1

- description Trunk-to-SW0

- switchport mode trunk

- switchport trunk encapsulation dot1q

- switchport trunk allowed vlan all

- exit

! --- HR PCs ---

- interface range fa0/2 - 3

- description HR-PCs

- switchport mode access

- switchport access vlan 10

- exit

! --- Save configuration ---

- end

- copy running-config startup-config

🔹 SW2 (IT Department Switch)

- enable

- configure terminal

! --- VLAN Creation (optional but recommended) ---

- vlan 20

- name IT

- exit

! --- Connection to SW0 (Fa0/1) ---

- interface fa0/1

- description Trunk-to-SW0

- switchport mode trunk

- switchport trunk encapsulation dot1q

- switchport trunk allowed vlan all

- exit

! --- IT PCs ---

- interface range fa0/2 - 3

- description IT-PCs

- switchport mode access

- switchport access vlan 20

- exit

! --- Save configuration ---

- end

- copy running-config startup-config

🧩 Verification Commands (after applying configs)

After configuration, use these to verify everything works correctly:

On all switches

show vlan brief

✅ Should show:

- VLAN 10 → Fa0/2, Fa0/3 on SW1

- VLAN 20 → Fa0/2, Fa0/3 on SW2

- VLAN 30 → Fa0/4, Fa0/5 on SW0

Check trunks

show interfaces trunk

✅ Should show trunk ports:

- SW0 → Fa0/1, Fa0/2, Fa0/3

- SW1 → Fa0/1

- SW2 → Fa0/1

Check link status

show interfaces status

✅ All connected ports should display connected.