Part A: Physical Topology Design & Device Setup

🔹 Step 1: Network Overview (Physical Layout Plan)

We’re simulating a 3-department enterprise:

| Department | VLAN | Subnet | Gateway | Devices |
| --- | --- | --- | --- | --- |
| HR | 10 | 192.168.10.0/24 | 192.168.10.1 | 2 PCs |
| IT | 20 | 192.168.20.0/24 | 192.168.20.1 | 2 PCs |
| Sales | 30 | 192.168.30.0/24 | 192.168.30.1 | 2 PCs |

Core Devices:

- 1 Router → Router-on-a-Stick (handles inter-VLAN routing)

- 3 Switches → one per department

- 6 PCs → 2 per VLAN

🔹 Step 2: Choose Device Models (Stable & Recommended)

These are the most stable Cisco Packet Tracer models that fully support VLANs, sub-interfaces, and trunking (avoid older models that miss VLAN trunk support):

| Device Type | Model | Interface Type | Notes |
| --- | --- | --- | --- |
| Router | Cisco 2911 | GigabitEthernet0/0, 0/1 | Supports sub-interfaces for VLANs |
| Switches | Cisco 2960 | FastEthernet ports | Ideal for VLANs & trunking |
| PCs | Generic PC | FastEthernet0 | DHCP/static IP configurable |

🔹 Step 3: Physical Topology in Packet Tracer

🔧 Connections

- Router (GigabitEthernet0/0) → Switch0 (Fa0/1)

  - Use a copper straight-through cable

  - Configure this as a trunk link

- Switch0 → Switch1 → Switch2

  - Connect them using Fa0/24 (trunk uplink ports)

  - Switch0 is the core switch

  - Switch1 and Switch2 connect downstream to departments

- PC Connections

  - HR PCs → Switch1 Fa0/1–2

  - IT PCs → Switch2 Fa0/1–2

  - Sales PCs → Switch0 Fa0/1–2

🧩 Topology Layout Example

[Router 2911]

|

(Trunk: G0/0 <-> Fa0/1)

|

[Switch0]  ← Core Switch

/        \

(Trunk Fa0/24)   (Trunk Fa0/23)

[Switch1]        [Switch2]

| |              | |

HR PCs           IT PCs

🔹 Step 4: Interface Connections Summary

| Connection | Cable Type | Port on Device A | Port on Device B | Mode |
| --- | --- | --- | --- | --- |
| Router ↔ Switch0 | Straight-through | G0/0 | Fa0/1 | Trunk |
| Switch0 ↔ Switch1 | Straight-through | Fa0/24 | Fa0/24 | Trunk |
| Switch0 ↔ Switch2 | Straight-through | Fa0/23 | Fa0/23 | Trunk |
| HR PC1 ↔ Switch1 | Straight-through | – | Fa0/1 | Access |
| HR PC2 ↔ Switch1 | Straight-through | – | Fa0/2 | Access |
| IT PC1 ↔ Switch2 | Straight-through | – | Fa0/1 | Access |
| IT PC2 ↔ Switch2 | Straight-through | – | Fa0/2 | Access |
| Sales PC1 ↔ Switch0 | Straight-through | – | Fa0/1 | Access |
| Sales PC2 ↔ Switch0 | Straight-through | – | Fa0/2 | Access |

🧱 Topology (as per your setup)

| Switch | Port | Connected To | Purpose |
| --- | --- | --- | --- |
| SW0 | Fa0/1 | Router G0/0 | Trunk (Router-on-a-Stick) |
| SW0 | Fa0/2 | SW1 Fa0/1 | Trunk |
| SW0 | Fa0/3 | SW2 Fa0/1 | Trunk |
| SW0 | Fa0/4 | PC1 | Access (Sales VLAN 30) |
| SW0 | Fa0/5 | PC2 | Access (Sales VLAN 30) |
| SW1 | Fa0/1 | SW0 Fa0/2 | Trunk |
| SW1 | Fa0/2 | PC3 | Access (HR VLAN 10) |
| SW1 | Fa0/3 | PC4 | Access (HR VLAN 10) |
| SW2 | Fa0/1 | SW0 Fa0/3 | Trunk |
| SW2 | Fa0/2 | PC5 | Access (IT VLAN 20) |
| SW2 | Fa0/3 | PC6 | Access (IT VLAN 20) |